Functional Document

1. Introduction

The Low-Field MRI Enhancement Pipeline project aims to improve the diagnostic quality of images produced by low-field MRI scanners. Low-field scanners operating at 0.1 to 0.5 Tesla produce images that are significantly noisier and blurrier compared to standard 1.5T or 3T machines. In the context of lumbar spine imaging, this degradation can blur disc edges, obscure nerve compression, and reduce fat-fluid contrast, all of which are critical for accurate diagnosis. Our system takes a fully classical, interpretable approach by simulating low-field characteristics from high-field data and then enhancing the simulated images through a structured multi-stage pipeline that runs on standard hardware without requiring GPUs.

2. Product Goal

The primary goal of this project is to develop a computationally efficient and fully interpretable classical enhancement pipeline for low-field MRI images. The system should be capable of processing over 100 patient volumes in an automated batch mode and produce enhanced images with measurable quality improvements as evaluated by standard metrics such as PSNR, SSIM, and SNR. The enhanced images should approach high-field perceptual quality while preserving anatomical structural integrity.

3. Demography (Users, Location)

Users

Target Users: Radiologists, clinicians, and medical imaging researchers

User Characteristics: Healthcare professionals working with MRI data in various clinical settings, ranging from well-equipped hospitals to resource-limited rural clinics

Location

Target Location: Healthcare facilities worldwide, with particular focus on resource-limited clinical settings where low-field MRI scanners are more commonly deployed

4. Business Processes

The key business processes include:

Data Acquisition and Preparation:

Process for collecting high-field MRI volumes and converting them from DICOM to NIfTI format while preserving spatial metadata and orientation information.

Low-Field Simulation:

Process for degrading high-field images to simulate realistic low-field characteristics using spatial resampling, point spread function blurring, and Rician noise injection.

Image Enhancement:

Process for applying a six-stage classical pipeline consisting of N4 bias correction, intensity standardization, Wiener deconvolution, resolution modeling, structural refinement, and SNR safety clamping to improve low-field image quality.

Evaluation and Reporting:

Process for computing quantitative quality metrics (PSNR, SSIM, SNR) by comparing enhanced volumes against high-field reference images and generating batch performance reports.

5. Features

This project implements the following key features:

Physics-Based Low-Field Simulation:

Simulate realistic low-field MRI from high-field source data using voxel resampling to 1.5 x 1.5 x 3.0 mm spacing, Gaussian PSF blurring with sigma 0.8, and calibrated Rician noise injection targeting 25 to 40 percent of the original high-field SNR.

Process 92 patient volumes in a single automated batch run.

Six-Stage Enhancement Pipeline:

Stage 1 — N4 bias field correction to remove intensity inhomogeneity.

Stage 2 — Intensity standardization to normalize brightness range.

Stage 3 — Wiener deconvolution for primary denoising and sharpening.

Stage 4 — Resolution modeling by blending sharp and smooth components.

Stage 5 — Structural refinement using unsharp masking for edge enhancement.

Stage 6 — SNR safety guard to clamp output within 70 to 90 percent of high-field reference.

Automated Batch Processing and Reporting:

Process 100+ patients unattended with per-patient metrics generation.

Generate filtered reports excluding suboptimal volumes and comparative PSNR/SSIM analysis.

6. Authorization Matrix

Define the roles and their corresponding access levels:

| Role | Access Level |
| --- | --- |
| Researcher | Full access to simulation, enhancement, and evaluation modules |
| Clinician | Access to enhanced output volumes and quality reports |
| Data Technician | Access to data preparation and format conversion tools |
| Reviewer | Read-only access to reports and evaluation metrics |

7. Assumptions

The high-field source data is available in standard 3D NIfTI format with correct spatial metadata and orientation information.

The computing environment has Python 3.13 with the required libraries (NiBabel, SimpleITK, scikit-image, SciPy) installed and configured.

The pipeline will be executed on standard hardware with a minimum of 8 GB RAM. No GPU or specialized hardware is required.

Team members possess the necessary understanding of MRI physics and image processing fundamentals to interpret and validate the results.